Objective 2 — Quantification of Next-Day Forecast Uncertainty

Objective of the Analysis

Quantify the uncertainty associated with the best-performing next-day solar PV forecasting model identified under Objective 1, using the differences between forecast and actual measured PV generation.

Data

- Actual measured PV generation for the out-of-sample evaluation period.

- Corresponding next-day predictions from the 1st-ranked forecasting model from Objective 1.

- Forecast and actual values must correspond to the same timestamps and forecasting intervals.

- Use only out-of-sample next-day forecasts when estimating uncertainty.

This follows the dissertation's distinction between the point forecast and the uncertainty surrounding that forecast.

Analysis to Perform

First calculate the forecast error for every observation:

et=Pactual,t-Pforecast,t

The distribution of these errors shall then be analysed to establish the magnitude and direction of the uncertainty associated with the selected forecasting model.

The analysis should quantify:

- Mean forecast error/bias (MBE) — establishes whether the model systematically over- or under-predicts generation.

- Standard deviation of forecast errors (σe) — measures the dispersion of actual generation around the forecast.

- Error percentiles/quantiles — establishes empirically how large forecast deviations are at selected confidence levels.

- Prediction/uncertainty bounds — determine the range within which actual generation can reasonably be expected to occur around a next-day forecast.

Where appropriate, uncertainty should also be expressed relative to forecast generation or plant capacity (%), so that the result can subsequently be applied to different forecast quantities.

Required Outputs

- Forecast-error dataset for the selected model.

- Summary statistics of the forecast errors.

- Forecast-error distribution/histogram.

- Actual versus forecast generation showing the derived uncertainty bounds.

- A table reporting the uncertainty associated with the selected next-day forecasting model.

The principal result for Objective 2 should take a form similar to:

| Uncertainty Measure | Result | Interpretation |
| --- | --- | --- |
| Mean Error / Bias | W | Direction of systematic forecast error |
| Error Standard Deviation | W | Typical dispersion around forecast |
| Relative Uncertainty | % | Uncertainty relative to forecast/capacity |
| 80% uncertainty interval | to | Expected forecast-error range |
| 90% uncertainty interval | to | Expected forecast-error range |
| 95% uncertainty interval | to | Expected forecast-error range |

Final Goal

Determine a quantitative and empirically derived measure of forecast uncertainty for the best-performing next-day forecasting model. The uncertainty measure shall be carried forward together with the next-day forecast into Objective 3, where market commitment quantities will be determined while considering forecast uncertainty and imbalance costs.